WIFI-ATBM6162 移植使用方法：

- 将x2670m-atbm6162-wifi6 文件夹下的文件移植到sdk/kernel/kernel5.10/对应的module_drivers/下面；

- 回到顶层目录make kernel-menuconfig 选中wifi配置，如图

- Location:

- -> Ingenic device-drivers Configurations

- -> [WIFI]  Drivers

Symbol: WLAN [=y]

Type  : bool

Defined at drivers/net/wireless/Kconfig:6

Prompt: Wireless LAN

Depends on: NETDEVICES [=y] && !S390 && NET [=y]

Location:

-> Device Drivers

-> Network device support (NETDEVICES [=y])

Selects: WIRELESS [=y]

Symbol: WIRELESS [=y]

Type  : bool

Defined at net/Kconfig:375

Prompt: Wireless

Depends on: NET [=y] && !S390

Location:

-> Networking support (NET [=y])

Selected by [y]:

- WLAN [=y] && NETDEVICES [=y] && !S390 && NET [=y]

Symbol: CRYPTO_USER_API_AEAD [=y]

Type  : tristate

Defined at crypto/Kconfig:1910

Prompt: User-space interface for AEAD cipher algorithms

Depends on: CRYPTO [=y] && NET [=y]

Location:

-> Cryptographic API (CRYPTO [=y])

Selects: CRYPTO_AEAD [=y] && CRYPTO_SKCIPHER [=y] && CRYPTO_NULL [=y] && CRYPTO_USER_API [=y]

- 编译ble 应用方法：

- 选中以上配置后,在驱动atbm_wifi6_40M_lite/路径下编译

- 直接编译命令： make ble_stack

- b. 修改驱动中的build_driver.sh 文件；

- elif [ "$1" = "x2600" ] || [ "$1" = "X2600" ];then

- KDIR=/home1/fwang/x2670-kernel5.10/kernel/kernel-5.10

- CC=/home1/fwang/x2670-kernel5.10/prebuilts/toolchains/mips-gcc-glibc/bin/mips-linux-gnu-

- architecture=mips

- system=Linux

- platform_soc=PLATFORM_INGENICX2600

- 将这两行红色字体的绝对路径改成当前文件sdk所在路径，命令：./build_driver.sh x2600

- 两种方法皆会生成以下文件：使用蓝牙时拷到板子端；

- 此时即可在顶层make kernel -j8 进行编译，kernel内置编译检查：

- kernel/kernel-5.10/module_drivers/drivers/net/wireless/atbm_wifi6_40M_lite/Makefie

- 直接将编译好的kernel文件烧录进去，内核启动时可以看到WIFI6的打印信息：

- 此时ifconfig -a 能看到wifi节点出现；

- 修改wifi配置文件 etc/wpa_supplicant.conf 的wifi名和密码，运行wifi_up.sh 脚本连接wifi；

# cat etc/wpa_supplicant.conf

#ctrl_interface=/var/run/wpa_supplicant

#ap_scan=1

#network={

#  key_mgmt=NONE

ctrl_interface=/var/run/wpa_supplicant

update_config=1

country=GB"

network={

ssid="WF"				//wifi名字

psk="asdfghjkl"			//wifi密码

bssid=

priority=1

}

- 连接bt 的方法：

- 将驱动中编译好的driver_install/nimble_linux  拷贝到板子上；

- 执行./nimble_linux & 命令；此时可以在手机蓝牙上搜索到设备，点击进行配对；

- 报错与解决办法：

- 检查kernel-menuconfig 没有选中ONLY_COMB_FIRMWARE；

kernel-menuconfig 选中CRYPTO_USER_API_AEAD、CRYPTO_USER_API_ENABLE_OBSOLETE;

- kernel-menuconfig 取消掉MAC80211